EngineeringDocs2LLM

Converter

Build Requirements & Technical Specification

Document Type: Technical Requirements Specification

Version: 1.0 | Date: March 2026

Classification: Internal

# 1. Executive Summary

EngineeringDocs2LLM Converter is a data ingestion and transformation platform that converts engineering documents (P&IDs, HAZOPs, LOPAs, and related process safety documentation) into structured, sensitivity-controlled formats optimized for Large Language Model (LLM) reasoning and analysis.

The system addresses a critical gap in process safety workflows: engineering documents contain rich structured data locked in proprietary formats (DWG, DXF) and semi-structured formats (PDF, XLSX) that LLMs cannot directly consume. The converter extracts this data, applies configurable sensitivity controls to protect intellectual property and security-sensitive information, and delivers it to LLM endpoints for automated analysis including deviation identification, safeguard adequacy review, LOPA verification, and gap analysis.

## 1.1 Key Capabilities

- Native DWG/DXF ingestion with structured entity extraction (blocks, attributes, layers, connectivity)

- PDF processing with text layer extraction + page rendering for LLM vision analysis

- Tabular document parsing (XLSX, CSV) with automatic HAZOP/LOPA field mapping

- Configurable sensitivity controls with five redaction strategies and hybrid cloud/on-prem routing

- Live LLM query interface with vision support for engineering drawing analysis

# 2. System Architecture

The system consists of four primary components deployed as containerized microservices behind an API gateway. The architecture separates the ingestion pipeline (which handles sensitive source files) from the LLM interface (which only receives sanitized payloads).

## 2.1 Component Overview

| Component | Technology | Responsibility |
| --- | --- | --- |
| CAD Processor | Python + ezdxf + ODA SDK | DWG/DXF parsing, entity extraction, block attribute resolution, layer filtering, topology graph construction |
| Document Processor | Python + pdfplumber + pdf2image | PDF text extraction, table extraction, page rendering to images, XLSX/CSV/DOCX parsing |
| Sensitivity Engine | Python/TypeScript | Field classification, redaction policy application (passthrough/abstract/tokenize/redact/noise), token registry management |
| LLM Gateway | TypeScript + API Router | Payload routing (cloud vs on-prem), context assembly, vision payload construction, response de-tokenization |
| Web Interface | React + TypeScript | Upload wizard, extraction verification, sensitivity configuration, LLM chat with vision |

## 2.2 Data Flow

Source Document (DWG/PDF/XLSX) → Ingestion Service → Structured Extraction → Sensitivity Transform → Routing Gate → Cloud or On-Prem LLM → Response Cache → De-tokenization → Analyst Review

The Token Registry (mapping opaque tokens to original values) is stored encrypted in a separate vault and is never transmitted to any LLM endpoint.

# 3. DWG/DXF Ingestion Pipeline

This is the highest-value and most technically complex component of the system. DWG files contain structured vector data with explicit entity relationships that are dramatically richer than what can be recovered from PDF or image-based processing.

## 3.1 Why DWG Over PDF

| Capability | DWG (Native) | PDF (Exported) |
| --- | --- | --- |
| Equipment Tags | Block attributes with explicit tag values (e.g., TAG=TK-4401) | Positioned text strings; must infer association to symbols via proximity |
| Layer Organization | Named layers: P-EQUIP, I-INST, P-PIPE, etc. Filter by discipline | Layers flattened; no discipline separation |
| Symbol Identity | Block definitions map to standard P&ID symbols (valve, pump, HX) | Symbols are geometry only; require vision/CV for identification |
| Connectivity | Entities share connection points; topology graph derivable from coordinates | Lines and shapes; connectivity must be inferred from intersection analysis |
| Instrument Data | Instrument blocks carry attributes: type, range, setpoint, tag | Text labels only; no structured association |
| Accuracy | 100% for attributes; connectivity depends on drawing quality | Vision: ~70-80% on clean drawings; text extraction: variable |

## 3.2 DWG Processing Architecture

### 3.2.1 Stage 1: Format Conversion

Tool: Open Design Alliance (ODA) File Converter / Teigha File Converter

Input: DWG files (all versions through AutoCAD 2026)

Output: DXF R2018 format + PDF rendering

DWG is a proprietary Autodesk binary format. Direct parsing requires licensing the RealDWG SDK from Autodesk (expensive, restrictive) or using the ODA SDK which provides a clean, well-documented API for reading DWG entities. The ODA SDK is free for non-commercial evaluation and reasonably priced for commercial use.

The converter outputs DXF (which ezdxf can parse natively) and optionally a PDF rendering (for the vision pipeline as a secondary verification channel).

### 3.2.2 Stage 2: Entity Extraction

Tool: ezdxf (Python, MIT license)

Installation: pip install ezdxf

ezdxf is the most mature open-source DXF processing library. It provides full access to the DXF entity model including blocks, inserts, attributes, layers, dimension entities, and spatial queries.

Key extraction operations:

- Block Definition Scan: Enumerate all BLOCK definitions. Map block names to P&ID symbol categories using a configurable lookup table (e.g., *PUMP* → Pump, *FCV* → Flow Control Valve).

- INSERT Traversal: Walk all INSERT entities in model space. For each insert, resolve the block reference, extract ATTRIB values (tag, service, spec), and record the insertion point coordinates.

- Layer Filtering: Group entities by layer name. Standard P&ID layers follow naming conventions (P- prefix for process, I- for instrument, E- for electrical). Use layer names to classify entity types.

- Connectivity Analysis: Build a topology graph by matching line/polyline endpoints to block insertion points within a configurable proximity threshold. This produces the from → to connectivity that's critical for process flow reasoning.

- Text Entity Extraction: Collect all TEXT and MTEXT entities. Associate floating text with nearest equipment/instrument block using spatial proximity. This captures annotations, notes, and specifications not stored as block attributes.

### 3.2.3 Stage 3: Structured Output

The extraction produces a structured JSON document containing equipment schedules, instrument indices, line lists, and a connectivity graph. This is the primary input to the sensitivity engine and is dramatically more informative for LLM reasoning than any vision-based or OCR-based extraction.

## 3.3 DWG Processing Code Reference

The following Python implementation demonstrates the core extraction logic using ezdxf. This is production-reference code, not pseudocode.

import ezdxf from ezdxf.math import Vec3 from collections import defaultdict import json def extract_pid_data(dxf_path: str, proximity: float = 5.0) -> dict: """Extract structured P&ID data from a DXF file.""" doc = ezdxf.readfile(dxf_path) msp = doc.modelspace() # 1. Extract all block inserts with attributes equipment = [] for insert in msp.query("INSERT"): attrs = {} for attrib in insert.attribs: attrs[attrib.dxf.tag] = attrib.dxf.text equipment.append({ "block_name": insert.dxf.name, "layer": insert.dxf.layer, "position": list(insert.dxf.insert), "attributes": attrs, "tag": attrs.get("TAG", attrs.get("TAGNO", "")), }) # 2. Build layer index layers = {} for layer in doc.layers: layers[layer.dxf.name] = { "color": layer.color, "entity_count": 0 } # 3. Extract line segments for connectivity lines = [] for entity in msp.query("LINE LWPOLYLINE POLYLINE"): layer = entity.dxf.layer if layer in layers: layers[layer]["entity_count"] += 1 if entity.dxftype() == "LINE": lines.append({ "start": list(entity.dxf.start), "end": list(entity.dxf.end), "layer": layer }) # 4. Build connectivity graph connections = [] for line in lines: start = Vec3(line["start"]) end = Vec3(line["end"]) from_equip = find_nearest(equipment, start, proximity) to_equip = find_nearest(equipment, end, proximity) if from_equip and to_equip: connections.append({ "from": from_equip["tag"], "to": to_equip["tag"], "line_layer": line["layer"] }) # 5. Extract text annotations annotations = [] for text in msp.query("TEXT MTEXT"): annotations.append({ "text": text.dxf.text if hasattr(text.dxf, "text") else text.text, "position": list(text.dxf.insert), "layer": text.dxf.layer }) return { "equipment": equipment, "connections": connections, "layers": layers, "annotations": annotations, "stats": { "total_equipment": len(equipment), "total_connections": len(connections), "total_layers": len(layers), } }

# 4. Build Requirements

## 4.1 Server Environment

| Requirement | Specification | Notes |
| --- | --- | --- |
| Operating System | Ubuntu 22.04 LTS or RHEL 9 | ODA SDK has Linux x64 binaries; container deployment recommended |
| Python | 3.10+ | Required for ezdxf, pdfplumber, pdf2image |
| Node.js | 20 LTS | Web interface and LLM gateway |
| Memory | 8 GB minimum, 16 GB recommended | Large DWG files can consume significant memory during parsing |
| Storage | 50 GB+ for document cache | Rendered PDF pages at 2x resolution ~2-5 MB each |
| GPU (optional) | NVIDIA A100/L40 for on-prem LLM | Only needed if running local model; not needed for cloud API |

## 4.2 Python Dependencies

| Package | Version | License | Purpose |
| --- | --- | --- | --- |
| ezdxf | 1.3+ | MIT | DXF parsing: entity traversal, block attributes, layer filtering, spatial queries |
| pdfplumber | 0.11+ | MIT | PDF text extraction, table detection and extraction |
| pdf2image | 1.17+ | MIT | PDF page rendering to PNG/JPEG for vision pipeline |
| poppler-utils | 22.02+ | GPL | System dependency for pdf2image (pdftoppm, pdftotext) |
| openpyxl | 3.1+ | MIT | XLSX reading and writing |
| python-docx | 1.1+ | MIT | DOCX text and table extraction |
| Pillow | 10.0+ | MIT-like | Image processing, format conversion, resizing |
| networkx | 3.2+ | BSD | Graph construction for P&ID connectivity analysis |
| cryptography | 42.0+ | Apache/BSD | AES-256 encryption for token registry and payloads at rest |
| anthropic | 0.40+ | MIT | Claude API client with vision support |

## 4.3 External Dependencies

| Component | Acquisition | Purpose |
| --- | --- | --- |
| ODA File Converter | opendesign.com (free eval / commercial license) | DWG → DXF conversion for all AutoCAD versions. CLI binary, no runtime dependency. Commercial license ~$2,500/yr. |
| Anthropic API Key | console.anthropic.com | Cloud LLM endpoint. Claude Sonnet recommended for vision analysis. Claude Opus for complex reasoning tasks. |
| vLLM or Ollama (optional) | Open source | On-prem LLM serving for high-sensitivity payloads. Llama 3.1 70B or Mistral Large recommended. |

## 4.4 Frontend Dependencies

| Package | Version | Purpose |
| --- | --- | --- |
| React | 18+ | UI framework |
| TypeScript | 5.3+ | Type safety across the entire frontend codebase |
| pdf.js | 3.11+ | Client-side PDF rendering (fallback for quick preview before server processing) |
| PapaParse | 5.4+ | Client-side CSV parsing for immediate preview |
| SheetJS | 0.20+ | Client-side XLSX parsing |

# 5. Processing Capabilities Matrix

This matrix defines exactly what the system can and cannot extract from each file format. This honesty is critical for setting correct expectations with engineering users.

| Capability | DWG/DXF | PDF | XLSX/CSV | Image |
| --- | --- | --- | --- | --- |
| Equipment Tags | Block attributes (100%) | Text layer (~90%) | Column data (100%) | Vision (~75%) |
| Symbol Identity | Block definitions (95%) | Vision only (~70%) | N/A | Vision (~70%) |
| Connectivity | Coordinate matching (90%) | Vision inference (~60%) | If column exists (100%) | Vision (~50%) |
| Process Conditions | Text/attribute (85%) | Text layer (~80%) | Column data (100%) | Vision (~60%) |
| Layer Organization | Full access (100%) | Lost in export | N/A | N/A |
| HAZOP Fields | N/A | Table extraction (~85%) | Full parsing (100%) | N/A |
| LOPA Frequencies | N/A | Table extraction (~85%) | Full parsing (100%) | N/A |

# 6. Deployment Architecture

## 6.1 Docker Containerization

The system deploys as three containers behind an nginx reverse proxy:

- cad-processor: Python 3.11 + ezdxf + ODA binary + pdfplumber + pdf2image. Handles all file ingestion. Exposes REST API for upload and extraction.

- llm-gateway: Node.js 20 + TypeScript. Sensitivity engine, routing logic, API proxy to cloud/on-prem LLM endpoints. Manages token registry.

- web-ui: React SPA served by nginx. Wizard interface for upload, extraction verification, sensitivity configuration, and LLM chat.

## 6.2 Docker Compose Reference

version: "3.8" services: cad-processor: build: ./services/cad-processor volumes: - ./data/uploads:/app/uploads - ./data/extracted:/app/extracted - ./oda-bin:/app/oda # ODA converter binary environment: - ODA_PATH=/app/oda/ODAFileConverter ports: ["8001:8001"] llm-gateway: build: ./services/llm-gateway volumes: - ./data/payloads:/app/payloads - ./data/tokens:/app/tokens # encrypted environment: - ANTHROPIC_API_KEY=${ANTHROPIC_API_KEY} - TOKEN_ENCRYPTION_KEY=${TOKEN_KEY} - ONPREM_LLM_URL=http://local-llm:8080 ports: ["8002:8002"] web-ui: build: ./services/web-ui ports: ["3000:80"] depends_on: [cad-processor, llm-gateway]

# 7. Security Architecture

## 7.1 Sensitivity Tiers

| Tier | Examples | Default Strategy | LLM Routing |
| --- | --- | --- | --- |
| PUBLIC | Equipment types, deviation categories | Pass Through | Cloud or On-Prem |
| INTERNAL | Equipment tags, line numbers, risk rankings | Abstract | Cloud (abstracted) or On-Prem |
| CONFIDENTIAL | Process conditions, chemical names, IPL PFDs | Tokenize | On-Prem only |
| RESTRICTED | Control setpoints, facility location, personnel | Redact | On-Prem only (if not redacted) |

## 7.2 Token Registry

The token registry is the most security-critical component. It maps opaque tokens back to original values and must be stored with the following controls:

- AES-256 encryption at rest with key stored in hardware security module or KMS

- Separate encryption key from payload encryption

- Access logging on every read operation

- Network isolation: never accessible from the same network path as any LLM endpoint

- Quarterly key rotation with backward-compatible decryption

## 7.3 Routing Policy

The routing gate inspects the _meta.max_residual_sensitivity field of each sanitized payload. If the maximum residual sensitivity (after all redaction strategies have been applied) is INTERNAL or below, the payload may be routed to a cloud LLM endpoint. If any field remains at CONFIDENTIAL or above after transformation, the payload must route to the on-premises model only.

# 8. Development Roadmap

| Phase | Duration | Deliverables | Dependencies |
| --- | --- | --- | --- |
| Phase 1 | 2-3 weeks | PDF + XLSX + CSV + DOCX ingestion pipeline with real extraction. Vision-based LLM chat. Web UI wizard. | Python env, Anthropic API key |
| Phase 2 | 3-4 weeks | DWG/DXF pipeline: ODA integration, ezdxf extraction, block-to-symbol mapping, connectivity graph builder. | ODA license, sample DWG files |
| Phase 3 | 2-3 weeks | Sensitivity engine: five redaction strategies, configurable presets, token registry with encryption, audit logging. | Security review, encryption keys |
| Phase 4 | 2-3 weeks | Hybrid routing: cloud vs on-prem decision gate, on-prem model deployment (vLLM + Llama 3.1 70B), response de-tokenization. | GPU hardware (if on-prem), network config |
| Phase 5 | 2-3 weeks | Docker containerization, CI/CD pipeline, production hardening, load testing, documentation. | Container registry, deployment environment |

Total estimated timeline: 11-16 weeks for full capability including DWG pipeline and hybrid LLM deployment.

Minimum viable product (Phase 1 only): 2-3 weeks for PDF/XLSX/vision-based analysis without DWG native support.

# 9. Cost Estimates

## 9.1 Infrastructure Costs (Annual)

| Item | Est. Cost/Year | Notes |
| --- | --- | --- |
| ODA File Converter License | $2,500 | Commercial license for DWG processing. Free for evaluation. |
| Anthropic API (Cloud LLM) | $600 - $3,000 | At occasional volume (~20 docs/month), Claude Sonnet with vision. Scales with usage. |
| GPU Server (On-Prem LLM) | $15,000 - $25,000 | One-time hardware cost for NVIDIA A100 80GB or L40S. Only if on-prem model required. |
| Application Server | $1,200 - $3,600 | Linux server with 16GB RAM. Cloud VM or on-prem. Runs Docker containers. |
| Total (Cloud-only) | $4,300 - $9,100 | Without on-prem GPU hardware |
| Total (Hybrid) | $19,300 - $34,100 | Including one-time GPU hardware purchase in year 1 |

## 9.2 Development Costs

Based on a senior full-stack engineer with Python CAD processing experience and LLM integration expertise, the estimated development effort is 440-640 hours across all five phases. At standard consulting rates, this represents $88,000-$192,000 in development costs depending on rate and complexity.

# 10. Acceptance Criteria

- DWG files from AutoCAD 2018-2026 parse without error and extract >90% of block attributes

- PDF P&IDs with text layers extract >85% of equipment tags and instrument tags

- XLSX/CSV HAZOPs and LOPAs parse with 100% fidelity for all populated cells

- Sensitivity engine correctly applies all five redaction strategies with reversible tokenization

- Token registry encryption passes security audit (AES-256, separate key, access logging)

- Routing gate correctly classifies payloads and routes to cloud vs on-prem based on residual sensitivity

- LLM provides accurate analysis of P&ID connectivity, HAZOP deviation coverage, and LOPA arithmetic from extracted data

- End-to-end processing time under 60 seconds for a typical 10-page P&ID DWG file

- System deploys via Docker Compose with a single command on a fresh Linux server